Assessment Tool 3

CO4_AT3_Comparative Analysis

Course: Computer Vision				Name: K.Teja Sri

Course Code: ITA0506					Register No:192525249

1. Object Detection Performance Comparison

Two object detection models, Viola–Jones and YOLO, produce the following results: Model A has 80 True Positives, 20 False Positives, and 30 False Negatives, while Model B has 90 True Positives, 15 False Positives, and 20 False Negatives.

(a) Calculate the precision and recall for both models.

(b) Compare the results and determine which model provides better overall object detection performance.

Justify your answer based on the calculated values.

1. Object Detection Performance Comparison

Given:

- Model A (Viola–Jones): TP = 80, FP = 20, FN = 30

- Model B (YOLO): TP = 90, FP = 15, FN = 20

(a) Precision and Recall

Precision formula:

Precision =   TP

TP+FP

Recall formula:

Recall = TP

TP+FN

Model A :

Precision A = 80   	= 80   =0.80

80+20   100

Precision = 80%

Recall A = 80  =  80  = 0.7273

80+30 110

Recall ≈ 72.73%

Model B :

Precision B= 90     = 90   = 0.8571

90+15   105

Precision ≈ 85.71%

Recall B = 90 =  90  = 0.8182

90+20  110

Recall ≈ 81.82%

(b) Comparison :

| Model | Precision | Recall |
| --- | --- | --- |
| Model A – Viola–Jones | 80% | 72.73% |
| Model B – YOLO | 85.71% | 81.82% |

Conclusion: Model B (YOLO) provides better overall object detection performance because it has higher precision and higher recall than Model A. This means YOLO produces fewer false detections and detects a larger proportion of the actual objects.

2. Motion Estimation Error Analysis

Two motion estimation techniques produce the following displacement errors in pixels: Method A: 2, 3, 2 and Method B: 3, 5, 4 for three consecutive frames.

(a) Calculate the average displacement error for both methods.

(b) Compare the two methods and determine which method provides more accurate motion estimation.

(c) Justify your answer based on the error values.

Given:

- Method A: 2, 3, 2 pixels

- Method B: 3, 5, 4 pixels

(a). Average Displacement Error

Method A:

Average ErrorA = 2+3+2 = 7  = 2.33 pixels

3        3

Method B:

Average ErrorB = 3+5+4 = 12  = 4 pixels

3           3

(b) Comparison :

| Method | Displacement Errors (pixels) | Average Error |
| --- | --- | --- |
| Method A | 2, 3, 2 | 2.33 pixels |
| Method B | 3, 5, 4 | 4 pixels |

Method A is more accurate because its average displacement error is lower.

(c) Justification

A smaller displacement error means that the estimated motion is closer to the actual motion. Method A has an average error of 2.33 pixels, whereas Method B has an average error of 4 pixels. Therefore, Method A provides more accurate motion estimation.

3. Depth Estimation Comparison

A stereo vision system and a Structure-from-Motion (SfM) system estimate the depth of three objects. The estimated depths are: Stereo Vision: 2.0 m, 3.0 m, 4.0 m and SfM: 2.5 m, 3.8 m, 5.0 m, while the actual depths are 2 m, 3 m, and 4 m, respectively.

(a) Calculate the absolute error for each object for both methods. (b) Calculate the average absolute error

for both methods.

(c) Compare the methods and identify which provides more accurate depth estimation.

Given:

| Object | Actual Depth | Stereo Vision | SfM |
| --- | --- | --- | --- |
| 1 | 2.0 m | 2.0 m | 2.5 m |
| 2 | 3.0 m | 3.0 m | 3.8 m |
| 3 | 4.0 m | 4.0 m | - m |

(a). Absolute Error:

Absolute Error formula:

Absolute Error=∣Estimated Depth−Actual Depth∣

Stereo Vision

Object 1:

∣2.0−2.0∣=0 m

Object 2:

∣3.0−3.0∣=0 m

Object 3:

∣4.0−4.0∣=0 m

SfM

Object 1:

∣2.5−2.0∣=0.5 m

Object 2:

∣3.8−3.0∣=0.8 m

Object 3:

∣5.0−4.0∣=1.0 m

Therefore:

| Object | Stereo Vision Error | SfM Error |
| --- | --- | --- |
| 1 | 0 m | 0.5 m |
| 2 | 0 m | 0.8 m |
| 3 | 0 m | 1.0 m |

(b) Average Absolute Error:

Stereo Vision:

Average Error = 0+0+0 = 0 m

3

SfM:

Average Error = 0.5+0.8+1.0 = 2.3 = 0.767 m

3	     3

(c) Comparison and Conclusion

| Method | Average Absolute Error |
| --- | --- |
| Stereo Vision | 0 m |
| SfM | 0.767 m |

Stereo Vision provides more accurate depth estimation because its average absolute error is 0 m, meaning its estimated depths exactly match the actual depths for all three objects. SfM has an average error of approximately 0.767 m, so it is less accurate for the given data.

Final Conclusion:

- Object Detection: Model B (YOLO) is better.

- Motion Estimation: Method A is better.

- Depth Estimation: Stereo Vision is better.